| Need for new Development | Effort |
| --- | --- |
| Load balancing and redundancy |  |
| User configured Port PWE are automatically distributed over available Resource cards in the system. |  |
| Upon failure of a resource card, the Port PWE configured over it are migrated to alternate Resource card |  |
| Traffic on a Port PWE to be load balanced across multiple Resource Cards/resource ports |  |
| support PUSHING new .1Q tag for traffic egress L2VPN attachment circuit. |  |
| Support for propagating QOS from EXP to .1p for end to end QOS |  |
| Support for registering to receive triggers based on MEP state. New work in CFM |  |
| Need RCM to map the Port PWE configuration into configuration for Resource port and L2VPN |  |
| RCM need to statically configure virtual ports (xc port and resource port) on resource card |  |
| Code changes in VXWorks code to bring up the loopback link between resource port and xc port on the resource card |  |
| Changes in FPGA programming to setup the loopback o resource card. |  |

| Current Limitation | Severity |
| --- | --- |
| NO support to load balance Port PWE traffic across multiple links |  |
| Can not support LAG of 2 10G Port PWE |  |
| Can not support Port PWE traffic containing customer vlans. |  |
| Egress queueing on the Resource Port will not be able to account congestion on LSP on the (L2 card) |  |
| Subscribers could be distributed across multiple Resource ports configured as Access LAG, in hitless mode. |  |